# Pseudomonas Minimal Medium (PMM)

Preparation of Broth and Agar formats

# Stock Solution Preparation

Sodium Succinate Solution (10% w/v):

- Dissolve 10.0 g of Sodium Succinate per 100 mL of dH2O.

- Sterilize via filter sterilization (0.22 µm membrane).

Ammonium Chloride (NH4Cl) Solution (20% w/v):

- Dissolve 20.0 g of NH4Cl per 100 mL of dH2O.

- Sterilize via filter sterilization (0.22 µm membrane).

Calcium Chloride (CaCl2) Stock Solution (1% w/v):

Requirement: 1.33% w/v of CaCl2·2H2O to account for hydration.

- Dissolve 0.665 g of CaCl2·2H2O in 50 mL of dH2O.

- Store at room temperature or 4°C.

# 2. Preparation Procedure (for 1.0 L Final Volume)

## Step A: Phosphate Buffer

- Add 900 mL of dH2O to a 1L flask.

- Dissolve 2.07 g of Monosodium Phosphate (NaH2PO4·H2O).

- Dissolve 2.61 g of Dipotassium Phosphate (K2HPO4).

- Adjust pH to 6.8 using 1N NaOH or HCl as needed.

## Step B: Mineral Base

- Add the following reagents to the buffer:

  - MgSO4: 0.5 g

  - FeCl3: 0.05 g

  - CaCl2 Stock Solution (1%): 0.5 mL

- Bring total volume up to 980 mL with dH2O.

Optional - Agar Addition. For solid media, add 15.0 g of agar.

- Sterilize the base (980 mL) via autoclave at 121°C for 20 minutes.

## Step C: Carbon and Nitrogen Source

After autoclaving, allow the medium to cool to ~50°C. In a laminar flow hood, add the filtered nutrients to complete the 1.0 L volume:

- 10.0 mL of Sodium Succinate (10% stock).

- 10.0 mL of NH4Cl (20% stock).